# Engagement Analysis: sF0KOVWj9p8

# Section 0: A person working on an electronics project

Time: 0:00 - 0:45

## Summary

The scene shows a person working on an electronics project, surrounded by various electronic devices and tools.

## Representative Frames

| 0:07 | 0:15 | 0:22 | 0:30 | 0:37 |
| --- | --- | --- | --- | --- |

## Transcript

[0:00] Sometimes when building circuits mistakes can happen, which can be a reverse voltage,
[0:11] a voltage applied to the wrong pin, components failing due to their age or wrong usage or
[0:17] you simply accidentally short something.
[0:20] No matter the cause, the now appearing overcurrent is the root of all evil that needs to get
[0:26] stopped so that nothing explodes or burns down completely.
[0:31] To do that I think pretty much everyone is familiar with such glass fuses that you just
[0:37] put in series to your circuit and as soon as too much current is flowing the thin metal
[0:43] inside of it melts and thus interrupts the current flow.

# Section 0: A person working on a circuit board

Time: 0:45 - 1:30

## Summary

The scene shows a person working on a circuit board, using an oscilloscope to measure a signal.

## Representative Frames

| 0:53 | 1:00 | 1:07 | 1:15 | 1:22 |
| --- | --- | --- | --- | --- |

## Transcript

[0:47] Only problem is that replacing such fuses can be cumbersome.
[0:51] It can get expensive if you are testing stuff and constantly destroy them and in case of
[0:57] enclosed electronics like your smartphone there is simply no way to replace such a fuse.
[1:04] The solution to all these problems though are resettable fuses like these ones which I often
[1:11] feel like not many people know about.
[1:14] So in this video I will show you exactly how to use them and with what limitations they
[1:20] come with.
[1:21] Because even though they are resettable and thus reusable they are certainly not perfect.

# Section 0: A person working on a project in a home or personal lab space

Time: 1:30 - 2:15

## Summary

The scene shows a person working on a project in a home or personal lab space, surrounded by various electronic devices and tools.

## Representative Frames

| 1:37 | 1:45 | 1:52 | 2:00 | 2:07 |
| --- | --- | --- | --- | --- |

## Transcript

[1:28] Let's get started!
[1:38] This video is sponsored by Mauser Electronics.
[1:42] Now truth be told I've been searching for fitting resettable fuses for a while on the internet.
[1:47] Mauser was the only seller that not only came with a big selection of them but most of them
[1:53] were also in stock, I could easily filter for preferences and best of all they sell to
[2:00] private people like you and me and not just companies.
[2:04] I have been ordering my components from them since 2018 and I can tell you for sure that they
[2:10] have all the components you could ever need.

# Section 0: A person working on a tabletop setup

Time: 2:15 - 3:00

## Summary

The scene shows a person working on a tabletop setup, surrounded by various electronic devices and tools.

## Representative Frames

| 2:22 | 2:30 | 2:37 | 2:45 | 2:52 |
| --- | --- | --- | --- | --- |

## Transcript

[2:14] And you usually receive them within 3 or sometimes even only 2 days.
[2:19] So feel free to check out Mauser Electronics if I piqued your interest.
[2:25] Now first off if you want to use resettable fuses then you always have the choice between the
[2:30] through hole ones that I feel like are easier to use and small SMD ones which you probably
[2:37] already have seen on devices like the Raspberry Pi.
[2:42] Sometimes resettable fuses are also known as a multi fuse, poly fuse or poly switch but
[2:48] if we want to be technically correct then they are actually called PPTC which stands for
[2:54] Polymeric Positive Temperature Coefficient device and wait a minute PTCs are actually quite

# Section 0: A person working on a project in an educational or experimental setting

Time: 3:00 - 3:45

## Summary

The scene shows a person working on a project in an educational or experimental setting, surrounded by various electronic devices and tools.

## Representative Frames

| 3:07 | 3:15 | 3:22 | 3:30 | 3:37 |
| --- | --- | --- | --- | --- |

## Transcript

[3:02] popular.
[3:04] What they do and what their name basically implies is that they increase their resistance when
[3:09] they get hotter.
[3:11] You can actually find their relative the NTC aka Negative Temperature Coefficient on lots
[3:17] of 3D printers because they decrease their resistance when their temperature rises and thus they both
[3:24] can be used to measure temperature.
[3:26] But back to the resettable fuse which does in fact increase its resistance when temperature
[3:31] rises but it still features lots of differences to normal PTCs or NTCs.
[3:38] So as a practical example let's say I got a circuit that normally draws around 500mA at