U5cqrrhq9TA

https://www.youtube.com/watch?v=U5cqrrhq9TA

Video ID: U5cqrrhq9TA | Category: Unknown Category | Topics model: llama3 | Topics created: 2025-10-09T17:32:46.298621+00:00

# Topics

## T1: Introduction

Overview of the topic and goals of the video.

## T2: Passive PFC vs. Active PFC

Explanation of passive and active PFCs and their differences.

## T3: Why Do We Need a PFC?

Discussion on the importance of power factor correction in AC-DC power supplies.

## T4: Practical Example: Using an Active PFC IC

Demonstration of how to use an active PFC IC with a few external components.

## T5: Conclusion

Summary and final thoughts on the topic of power factor correction.

# Transcript (plain)

This heat gun is pure perfection while this LED strip is straight up garbage. Wait, what? The reason why I am saying this is because when looking at the current consumption of those two loads we can see a remarkable difference. First off the heat gun which draws current exactly in phase with our mains voltage and its shape also almost looks like it aka sinusoidal. Lovely. But next we got the terrible LED strip which draws current abruptly and only for a short amount of time near the peak of our mains voltage. Okay so what is the problem here? I mean both devices seem to work perfectly fine. Well the problem is that the LED strip with its AC to DC power supply draws a lot of apparent power from the power grid because of the weird current waveform while the real power it actually requires to do its job of lighting up LEDs is rather small in comparison. That is a huge problem because nowadays we got tons of AC to DC power supplies in our homes and if they would all perform this bad then our power grids will have to face some big challenges in the future. But it doesn't have to be this way and that is why such power factor correction circuits or PFC for short do exist. And in this video I will show you what they can do, how we can make them and why they are super important for our electrical future. Let's get started! This video is sponsored by Mauser Electronics who not only offer a huge selection of electronics components but also development boards. And I am not talking about microcontroller boards or similar even though they also have them. No, I am talking about development boards built around the newest ICs and technologies which also includes functional PFCs that you will see later. If you are curious for more then head over to Mauser Electronics or check the link in the video description. Now let's start off with a popular analogy which is the foam to drink ratio. You see the liquid alone represents our real power aka the power that actually gets used to light things up, heat things up or for example charge your phone's battery. But as you can see that is not all in our glass which in total represents our apparent power and that is the power your energy provider actually has to deal with in its grid so that you can get your desired real power. No, we still got the foam which is our reactive power and truth be told completely useless because it only oscillates between the energy provider and hooked up loads. Due to it more current flows through the grid than what we actually need for the real power demand and as you might know for bigger current flow we need thicker wires so that things do not get terribly inefficient and hot and that of course costs lots of money. So keeping the reactive power close to zero is our goal here meaning that apparent power should equal the real power. And in the heat gun example we started with we can actually see such a behaviour that real power equals apparent power meaning it is definitely possible. Which brings us to the question how such reactive power comes to be. The most popular examples for that are simply a motor and a capacitive power supply that you can sometimes find in LED lights. Both of them create quite a bit of reactive power in comparison to their real power power and the root of all evil for that is like I teased at the beginning their current flow. The power supply ones leads in comparison to the mains voltage and the one from the motor lacks in comparison. The reason for that are of course the electrical properties of the capacitor for the power supply and the inductance for the motor coils. But feel free to watch my previous videos about those topics here so that I can keep the rest of this video short and sweet. Because in a nutshell such phase shifts do create reactive power. And the simple fix here is to add inductance to the capacitance and vice versa capacitance to the inductance to get rid of the phase shifts and thus reduce the reactive power and get a power factor of almost 1. By the way the power factor is simply the real power divided by the apparent power and thus 1 means real power equals apparent power which is what we are after. Now like I said one solution is adding capacitance or inductance to the power grid to decrease the phase shifts and that is actually the first type of a power factor correction circuit and it is called a passive one. Of course there is also an active one that I already presented to you at the start. And to understand why we need it let's go back to the current draw of a common power supply I found in my shelf. As you can see the more DC current I draw from its output the bigger the current pulse on the input near the peak of the mains voltage gets. And by doing my usual measurements next we can see that this current draw creates kind of a lot of reactive power which is easily noticeable through the low power factor. But there is no phase shift so how does that work? Well the problem here is that this current waveform can be depicted as a sum of lots of current waveforms with different frequencies. Most importantly of course is the 50Hz fundamental frequency which is our mains voltage frequency and it obviously does not feature a phase shift. But then there are also 150Hz, 250Hz, 350Hz and so on parts which are called 3rd, 5th, 7th and so on current harmonics and they do kind of feature a phase shift that creates the reactive power. By using my oscilloscope we can actually see how much they matter in comparison to the fundamental 50Hz frequency and let's just say that they are pretty dominant on the screen here. So what we can do to fix this problem is basically convert this small current pulse to an even sinusoidal one. And like I said before a circuit that can do that is an active PFC for which I looked for on Mauser electronics and I found a promising looking IC that only requires a few external components to function. But before creating a whole PCB for it which requires lots of time I firstly wanted to do some initial tests with a similar IC for which Mauser luckily offered a development board. After powering its low voltage side with my lab bench power supply I next prepared a proper mains voltage wire for it and this break might be a good opportunity to state that working with all the mains voltage shown in this video can be dangerous and you should only do that if you are professional. Okay with that out of the way I plugged the circuit into mains voltage and was very happy to find out that nothing exploded. On its output I measured a voltage of around 400VDC which means the circuit works correctly. And before I start explaining here why this high voltage and how it is made and what not, let's rather keep it practical and crack open the power supply that came with the horrible current draw. After removing its full bridge rectifier, I simply connected the output of the PFC to the input of the power supply and by once again plugging in mains voltage nothing blew up and the power supply seems to work correctly as well. This time though the input current does follow the mains voltage shape a lot more when drawing more current on the outputs. That not only means that the current harmonics are mostly gone but also that the reactive power of the system decreased while the power factor increased. Awesome! I think you can see the difference with and without a PFC pretty clearly in this chart here. Which now brings us to the question how such an active PFC pulls this off? Well in a nutshell it does so by increasing the changing mains input voltage into a constant higher DC voltage voltage that the power supply then uses. Because yes, an active PFC is basically just a glorified boost converter. You see the problem of normal AC to DC power supplies is that they only draw current near the voltage peak of the mains voltage because that is when the utilized capacitor's voltage drops beneath the input voltage and thus can get charged up again. But by supplying a constant boosted voltage this problem basically disappears. Of course creating such a stable boosted DC voltage from a varying input voltage is not that easy because you need to monitor the input and output voltage as well as the flowing current current and then turn on or off the boost converter MOSFET switch accordingly so that always the right amount of energy gets transferred to the output. When having a look at our practical example then the MOSFET switching pattern looks something like this with a bigger duty cycle at low input voltages and a smaller duty cycle at higher input voltages which corresponds with the theory. So all in all such active PFC are pretty awesome. And considering that there are already regulations in place that limit the created current harmonics from appliances they will sooner or later become mandatory. So if you are shopping for a new beefy power supply maybe next time consider getting one with a built in PFC. With that being said I hope you enjoyed this video and learn something new. As always don't forget to like, share, subscribe and hit the notification bell. Stay creative and I will see you next time!

# Transcript (timestamped)

[00:00:00.000 → 00:00:10.560] This heat gun is pure perfection while this LED strip is straight up garbage.

[00:00:10.560 → 00:00:17.880] Wait, what? The reason why I am saying this is because when looking at the current consumption

[00:00:17.880 → 00:00:24.540] of those two loads we can see a remarkable difference. First off the heat gun which draws

[00:00:24.540 → 00:00:31.300] current exactly in phase with our mains voltage and its shape also almost looks like it aka

[00:00:31.300 → 00:00:39.840] sinusoidal. Lovely. But next we got the terrible LED strip which draws current abruptly and

[00:00:39.840 → 00:00:45.040] only for a short amount of time near the peak of our mains voltage.

[00:00:45.040 → 00:00:52.300] Okay so what is the problem here? I mean both devices seem to work perfectly fine.

[00:00:52.300 → 00:00:58.880] Well the problem is that the LED strip with its AC to DC power supply draws a lot of apparent

[00:00:58.880 → 00:01:05.720] power from the power grid because of the weird current waveform while the real power it actually

[00:01:05.720 → 00:01:11.980] requires to do its job of lighting up LEDs is rather small in comparison.

[00:01:11.980 → 00:01:18.800] That is a huge problem because nowadays we got tons of AC to DC power supplies in our homes

[00:01:18.800 → 00:01:23.940] and if they would all perform this bad then our power grids will have to face some big

[00:01:23.940 → 00:01:30.500] challenges in the future. But it doesn't have to be this way and that is why such power factor

[00:01:30.500 → 00:01:38.240] correction circuits or PFC for short do exist. And in this video I will show you what they can do,

[00:01:38.240 → 00:01:45.680] how we can make them and why they are super important for our electrical future. Let's get started!

[00:01:48.800 → 00:02:00.080] This video is sponsored by Mauser Electronics who not only offer a huge selection of electronics

[00:02:00.080 → 00:02:07.120] components but also development boards. And I am not talking about microcontroller boards or similar

[00:02:07.120 → 00:02:14.400] even though they also have them. No, I am talking about development boards built around the newest ICs

[00:02:14.400 → 00:02:22.880] and technologies which also includes functional PFCs that you will see later. If you are curious for more

[00:02:22.880 → 00:02:27.600] then head over to Mauser Electronics or check the link in the video description.

[00:02:29.280 → 00:02:37.600] Now let's start off with a popular analogy which is the foam to drink ratio. You see the liquid alone

[00:02:37.600 → 00:02:45.280] represents our real power aka the power that actually gets used to light things up, heat things up or

[00:02:45.280 → 00:02:52.400] for example charge your phone's battery. But as you can see that is not all in our glass which in total

[00:02:52.400 → 00:02:59.280] represents our apparent power and that is the power your energy provider actually has to deal with in its grid

[00:02:59.280 → 00:03:07.280] so that you can get your desired real power. No, we still got the foam which is our reactive power

[00:03:07.920 → 00:03:15.840] and truth be told completely useless because it only oscillates between the energy provider and hooked up loads.

[00:03:16.880 → 00:03:21.920] Due to it more current flows through the grid than what we actually need for the real power demand

[00:03:21.920 → 00:03:29.600] and as you might know for bigger current flow we need thicker wires so that things do not get terribly

[00:03:29.600 → 00:03:37.920] inefficient and hot and that of course costs lots of money. So keeping the reactive power close to zero

[00:03:37.920 → 00:03:45.760] is our goal here meaning that apparent power should equal the real power. And in the heat gun example we

[00:03:45.760 → 00:03:52.960] started with we can actually see such a behaviour that real power equals apparent power meaning it is

[00:03:52.960 → 00:03:58.800] definitely possible. Which brings us to the question how such reactive power comes to be.

[00:04:00.000 → 00:04:06.320] The most popular examples for that are simply a motor and a capacitive power supply that you can

[00:04:06.320 → 00:04:14.080] sometimes find in LED lights. Both of them create quite a bit of reactive power in comparison to their real power

[00:04:14.080 → 00:04:20.640] power and the root of all evil for that is like I teased at the beginning their current flow.

[00:04:21.600 → 00:04:28.640] The power supply ones leads in comparison to the mains voltage and the one from the motor lacks in comparison.

[00:04:29.520 → 00:04:34.640] The reason for that are of course the electrical properties of the capacitor for the power supply

[00:04:35.200 → 00:04:42.160] and the inductance for the motor coils. But feel free to watch my previous videos about those topics here

[00:04:42.160 → 00:04:49.120] so that I can keep the rest of this video short and sweet. Because in a nutshell such phase shifts

[00:04:49.120 → 00:04:56.480] do create reactive power. And the simple fix here is to add inductance to the capacitance and vice versa

[00:04:56.480 → 00:05:03.680] capacitance to the inductance to get rid of the phase shifts and thus reduce the reactive power

[00:05:03.680 → 00:05:11.440] and get a power factor of almost 1. By the way the power factor is simply the real power divided by the

[00:05:11.440 → 00:05:18.400] apparent power and thus 1 means real power equals apparent power which is what we are after.

[00:05:19.440 → 00:05:25.760] Now like I said one solution is adding capacitance or inductance to the power grid to decrease the

[00:05:25.760 → 00:05:33.520] phase shifts and that is actually the first type of a power factor correction circuit and it is called a

[00:05:33.520 → 00:05:40.560] passive one. Of course there is also an active one that I already presented to you at the start.

[00:05:41.440 → 00:05:47.920] And to understand why we need it let's go back to the current draw of a common power supply I found in my shelf.

[00:05:48.960 → 00:05:55.440] As you can see the more DC current I draw from its output the bigger the current pulse on the input

[00:05:55.440 → 00:06:02.640] near the peak of the mains voltage gets. And by doing my usual measurements next we can see that this

[00:06:02.640 → 00:06:09.760] current draw creates kind of a lot of reactive power which is easily noticeable through the low power factor.

[00:06:10.640 → 00:06:18.480] But there is no phase shift so how does that work? Well the problem here is that this current waveform

[00:06:18.480 → 00:06:23.200] can be depicted as a sum of lots of current waveforms with different frequencies.

[00:06:24.240 → 00:06:31.920] Most importantly of course is the 50Hz fundamental frequency which is our mains voltage frequency and

[00:06:31.920 → 00:06:42.640] it obviously does not feature a phase shift. But then there are also 150Hz, 250Hz, 350Hz and so on parts

[00:06:43.200 → 00:06:50.800] which are called 3rd, 5th, 7th and so on current harmonics and they do kind of feature a phase shift

[00:06:50.800 → 00:06:58.160] that creates the reactive power. By using my oscilloscope we can actually see how much they matter in comparison

[00:06:58.160 → 00:07:05.280] to the fundamental 50Hz frequency and let's just say that they are pretty dominant on the screen here.

[00:07:06.320 → 00:07:13.360] So what we can do to fix this problem is basically convert this small current pulse to an even sinusoidal

[00:07:13.360 → 00:07:21.680] one. And like I said before a circuit that can do that is an active PFC for which I looked for on Mauser

[00:07:21.680 → 00:07:28.880] electronics and I found a promising looking IC that only requires a few external components to function.

[00:07:29.920 → 00:07:36.720] But before creating a whole PCB for it which requires lots of time I firstly wanted to do some

[00:07:36.720 → 00:07:42.640] initial tests with a similar IC for which Mauser luckily offered a development board.

[00:07:42.640 → 00:07:49.760] After powering its low voltage side with my lab bench power supply I next prepared a proper mains

[00:07:49.760 → 00:07:56.560] voltage wire for it and this break might be a good opportunity to state that working with all the

[00:07:56.560 → 00:08:03.600] mains voltage shown in this video can be dangerous and you should only do that if you are professional.

[00:08:04.640 → 00:08:11.680] Okay with that out of the way I plugged the circuit into mains voltage and was very happy to find out

[00:08:11.680 → 00:08:20.800] that nothing exploded. On its output I measured a voltage of around 400VDC which means the circuit works correctly.

[00:08:21.920 → 00:08:27.520] And before I start explaining here why this high voltage and how it is made and what not,

[00:08:28.160 → 00:08:34.560] let's rather keep it practical and crack open the power supply that came with the horrible current draw.

[00:08:35.600 → 00:08:41.360] After removing its full bridge rectifier, I simply connected the output of the PFC

[00:08:41.360 → 00:08:48.880] to the input of the power supply and by once again plugging in mains voltage nothing blew up and the

[00:08:48.880 → 00:08:55.760] power supply seems to work correctly as well. This time though the input current does follow the mains

[00:08:55.760 → 00:09:02.720] voltage shape a lot more when drawing more current on the outputs. That not only means that the current

[00:09:02.720 → 00:09:09.920] harmonics are mostly gone but also that the reactive power of the system decreased while the power factor

[00:09:09.920 → 00:09:18.000] increased. Awesome! I think you can see the difference with and without a PFC pretty clearly in this chart here.

[00:09:18.720 → 00:09:23.040] Which now brings us to the question how such an active PFC pulls this off?

[00:09:24.160 → 00:09:30.720] Well in a nutshell it does so by increasing the changing mains input voltage into a constant higher

[00:09:30.720 → 00:09:39.520] DC voltage voltage that the power supply then uses. Because yes, an active PFC is basically just a glorified

[00:09:39.520 → 00:09:47.520] boost converter. You see the problem of normal AC to DC power supplies is that they only draw current near

[00:09:47.520 → 00:09:54.000] the voltage peak of the mains voltage because that is when the utilized capacitor's voltage drops beneath

[00:09:54.000 → 00:10:02.320] the input voltage and thus can get charged up again. But by supplying a constant boosted voltage this problem

[00:10:02.320 → 00:10:09.920] basically disappears. Of course creating such a stable boosted DC voltage from a varying input voltage

[00:10:09.920 → 00:10:17.200] is not that easy because you need to monitor the input and output voltage as well as the flowing current

[00:10:17.200 → 00:10:23.920] current and then turn on or off the boost converter MOSFET switch accordingly so that always the right

[00:10:23.920 → 00:10:31.520] amount of energy gets transferred to the output. When having a look at our practical example then the MOSFET

[00:10:31.520 → 00:10:38.560] switching pattern looks something like this with a bigger duty cycle at low input voltages and a smaller

[00:10:38.560 → 00:10:47.120] duty cycle at higher input voltages which corresponds with the theory. So all in all such active PFC

[00:10:47.120 → 00:10:54.880] are pretty awesome. And considering that there are already regulations in place that limit the created current

[00:10:54.880 → 00:11:02.960] harmonics from appliances they will sooner or later become mandatory. So if you are shopping for a new beefy power supply

[00:11:03.600 → 00:11:11.200] maybe next time consider getting one with a built in PFC. With that being said I hope you enjoyed this video

[00:11:11.200 → 00:11:18.880] and learn something new. As always don't forget to like, share, subscribe and hit the notification bell.

[00:11:19.920 → 00:11:24.880] Stay creative and I will see you next time!